|  | BEŞ ELEKTRİK ENERJİ MAKİNE A.Ş. (TURKEY) |
| --- | --- |
|  | Gayrettepe Mah.Yıldız Posta Caddesi Akın Sitesi 8/13 Ofis:5 Gayrettepe Mah./İstanbul/Beşiktaş www.5enerji.com.tr Email: cetinakcen@gmail.com55sda Tel: + 90 546 650 21 95 Tel: + 90 535 020 70 59 |

| Model | 5E-WT3-K/N | 5E-WT5-K/N | 5E-WT10-K/N |
| --- | --- | --- | --- |
| Orientation | Upwind | Upwind | Upwind |
| Rated Output | 3kw | 5kW | 10kW |
| Peak Output | 4kw | 6,5kW | 11,5kW |
| Output Voltage (V) | 12-24-220-380(Optional) | 24-48-220-380V(Optional) | 24-48-220-380V(Optional) |
| Generator Type | Synchronous Direct Drive | Synchronous Direct Drive | Synchronous Direct Drive |
| Applications | Off/On Grid Connection | Off/On Grid Connection | Off/On Grid Connection |
| Controller Type | PLC | PLC | PLC |
| Overspeed Protection | Blade Tip Brakes with Pitch Control | Blade Tip Brakes with Pitch Control | Blade Tip Brakes with Pitch Control |
| Blade Material | Carbonfiber | Carbonfiber | Carbonfiber |
| Number of Blades | 3 | 3 | 3 |
| Rotor Diameter (m) | 2,7 | 3,5 | 6 |
| Swept Area (m2) | 5,7 | 9,6 | 30,5 |
| Windspeed (m/s) |  |  |  |
| Rated | 13 | 13 | 13 |
| Cut-in | 3 | 3 | 3 |
| Cut-out | 18 | 18 | 18 |
| Governing |  |  |  |
| Survival | Pitch Control Max. 23 m/sec | Pitch Control Max. 23 m/sec | Pitch Control Max. 23 m/sec |
| Head Weight (kg) | 75 | 150 | 300 |
| Tower Type | Steel | Steel | Steel |
| Tower Height (m) | 12-18 | 12-18 | 12-18 |
| Product Life (years) | 25 | 25 | 25 |
| Warranty (years) | 2 | 2 | 2 |
| Units sold | 3 | 15 | 20 |
| On the market since | 5 | 5 | 5 |
| Price | Contact 5 Energy | Contact 5 Energy | Contact 5 Energy |
| Certificate | TSE-61400 | TSE-61400 | TSE-61400 |